|  | COMSATS University Islamabad, Lahore Campus Department of Computer Engineering |
| --- | --- |

Embedded System Workshop

Lab Submission

Lab 1, Lab 2 and Lab 3

Program Codes, In-Lab Tasks and Lab Tasks

Group Members:

| Sr. No. | Name | Registration No. |
| --- | --- | --- |
| 1 | Uzair | FA23-BCE-098 |

| Program: |  | Computer Engineering |
| --- | --- | --- |
| Course: |  | Embedded System Workshop |
| Course Instructor: |  |  |
| Date of Submission: |  |  |

# Lab1

## Program1

#include "driver/gpio.h"

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#define LED_PIN GPIO_NUM_2 // Define the GPIO pin for the LED

#define ESP_INTR_FLAG_DEFAULT 0

#define Pause 250 //delay of ms

// Delay for milliseconds

void delay_ms(unsigned int sec){

vTaskDelay (sec / portTICK_PERIOD_MS);

}

// Function to initialize GPIO

void initialize_gpio()

{

// Configure GPIOs for LED

gpio_config_t io_conf_led;

io_conf_led.intr_type = GPIO_INTR_DISABLE; // Disable interrupts

io_conf_led.mode = GPIO_MODE_OUTPUT; // Set as output mode

io_conf_led.pin_bit_mask = (1ULL << LED_PIN); // Set the pin mask

io_conf_led.pull_down_en = GPIO_PULLDOWN_DISABLE; // Disable pull-down resistor

io_conf_led.pull_up_en = GPIO_PULLUP_DISABLE; // Disable pull-up resistor

gpio_config(&io_conf_led);

}

void app_main(void){

initialize_gpio();

while (1) {

gpio_set_level(LED_PIN, 1); // Turn LED on

delay_ms(3*Pause);

gpio_set_level(LED_PIN, 0); // Turn LED off

delay_ms(Pause);

}

}

## Program 2

#include "driver/gpio.h"

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#define PB_PIN GPIO_NUM_18 // Define the GPIO pin for the Pushbutton

#define ESP_INTR_FLAG_DEFAULT 0

// Function to initialize GPIO

void initialize_gpio(){

gpio_config_t pb_input;

// Configure the GPIO as input

pb_input.intr_type = GPIO_INTR_DISABLE;

pb_input.mode = GPIO_MODE_INPUT;

pb_input.pin_bit_mask = (1ULL<<PB_PIN);

pb_input.pull_down_en = GPIO_PULLDOWN_DISABLE;

pb_input.pull_up_en = GPIO_PULLUP_DISABLE;

gpio_config(&pb_input);

}

// Delay for milliseconds

void delay_ms(unsigned int ms){

vTaskDelay(pdMS_TO_TICKS(ms));

}

//********************************************************************************

void app_main(void){

// Initialize GPIO

initialize_gpio();

while (1) {

// Read the state of the GPIO

int level = gpio_get_level(PB_PIN); //read pushbutton status

}

}

## In Lab Exercise 1

#include "driver/gpio.h"

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#define LED_PIN GPIO_NUM_2 // Define the GPIO pin for the LED

#define ESP_INTR_FLAG_DEFAULT 0

#define Pause 250 //delay of ms

// Delay for milliseconds

void delay_ms(unsigned int sec){

vTaskDelay (sec / portTICK_PERIOD_MS);

}

// Function to initialize GPIO

void initialize_gpio()

{

// Configure GPIOs for LED

gpio_config_t io_conf_led;

io_conf_led.intr_type = GPIO_INTR_DISABLE; // Disable interrupts

io_conf_led.mode = GPIO_MODE_OUTPUT; // Set as output mode

io_conf_led.pin_bit_mask = (1ULL << LED_PIN); // Set the pin mask

io_conf_led.pull_down_en = GPIO_PULLDOWN_DISABLE; // Disable pull-down resistor

io_conf_led.pull_up_en = GPIO_PULLUP_DISABLE; // Disable pull-up resistor

gpio_config(&io_conf_led);

}

void app_main(void){

initialize_gpio();

while (1) {

gpio_set_level(LED_PIN, 1); // Turn LED on

delay_ms(500);

gpio_set_level(LED_PIN, 0); // Turn LED off

delay_ms(200);

}

}

## In Lab Exercise 2

#include "driver/gpio.h"

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#define LED_PIN GPIO_NUM_2      // GPIO pin for the LED

#define LED_ON_TIME_MS  500     // LED on duration in ms

#define LED_OFF_TIME_MS 200     // LED off duration in ms

void delay_ms(unsigned int ms)

{

vTaskDelay(ms / portTICK_PERIOD_MS);

}

void initialize_gpio(void)

{

gpio_config_t io_conf_led = {

.intr_type    = GPIO_INTR_DISABLE,   // Disable interrupts

.mode         = GPIO_MODE_OUTPUT,    // Set as output

.pin_bit_mask = (1ULL << LED_PIN),   // Pin mask

.pull_down_en = GPIO_PULLDOWN_DISABLE,

.pull_up_en   = GPIO_PULLUP_DISABLE,

};

gpio_config(&io_conf_led);

}

void app_main(void)

{

initialize_gpio();

while (1) {

gpio_set_level(LED_PIN, 1);   // LED on

delay_ms(LED_ON_TIME_MS);     // 500 ms

gpio_set_level(LED_PIN, 0);   // LED off

delay_ms(LED_OFF_TIME_MS);    // 200 ms

}

}

## Lab Task 1

#include "driver/gpio.h"

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#define LED1_PIN GPIO_NUM_18

#define LED2_PIN GPIO_NUM_19

#define LED3_PIN GPIO_NUM_21

#define INTERVAL_MS 250   // Time between each step

// Delay for milliseconds

void delay_ms(unsigned int ms)

{

vTaskDelay(ms / portTICK_PERIOD_MS);

}

// Initialize all three LED pins as outputs

void initialize_gpio(void)

{

gpio_config_t io_conf = {

.intr_type    = GPIO_INTR_DISABLE,

.mode         = GPIO_MODE_OUTPUT,

.pin_bit_mask = (1ULL << LED1_PIN) | (1ULL << LED2_PIN) | (1ULL << LED3_PIN),

.pull_down_en = GPIO_PULLDOWN_DISABLE,

.pull_up_en   = GPIO_PULLUP_DISABLE,

};

gpio_config(&io_conf);

}

// Set the state of all three LEDs

void set_leds(int l1, int l2, int l3)

{

gpio_set_level(LED1_PIN, l1);

gpio_set_level(LED2_PIN, l2);

gpio_set_level(LED3_PIN, l3);

}

void app_main(void)

{

initialize_gpio();

while (1) {

set_leds(1, 1, 0);   // LED1 + LED2 on

delay_ms(INTERVAL_MS);

set_leds(0, 1, 1);   // LED2 + LED3 on

delay_ms(INTERVAL_MS);

set_leds(1, 0, 1);   // LED1 + LED3 on

delay_ms(INTERVAL_MS);

}

}

## Lab Task 2

#include "driver/gpio.h"

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#define LED_PIN     GPIO_NUM_2    // On-board LED

#define BUTTON_PIN  GPIO_NUM_18   // Pushbutton input

#define POLL_DELAY_MS 10          // Polling interval (also gives basic debounce)

// Delay for milliseconds

void delay_ms(unsigned int ms)

{

vTaskDelay(ms / portTICK_PERIOD_MS);

}

// Initialize LED as output and button as input with pull-up

void initialize_gpio(void)

{

// LED configuration

gpio_config_t led_conf = {

.intr_type    = GPIO_INTR_DISABLE,

.mode         = GPIO_MODE_OUTPUT,

.pin_bit_mask = (1ULL << LED_PIN),

.pull_down_en = GPIO_PULLDOWN_DISABLE,

.pull_up_en   = GPIO_PULLUP_DISABLE,

};

gpio_config(&led_conf);

// Button configuration

gpio_config_t btn_conf = {

.intr_type    = GPIO_INTR_DISABLE,

.mode         = GPIO_MODE_INPUT,

.pin_bit_mask = (1ULL << BUTTON_PIN),

.pull_down_en = GPIO_PULLDOWN_DISABLE,

.pull_up_en   = GPIO_PULLUP_ENABLE,    // Pin reads 1 when released

};

gpio_config(&btn_conf);

}

void app_main(void)

{

initialize_gpio();

while (1) {

// Button is active-low: pressed = 0, released = 1

if (gpio_get_level(BUTTON_PIN) == 0) {

gpio_set_level(LED_PIN, 1);   // Pressed: LED on

} else {

gpio_set_level(LED_PIN, 0);   // Released: LED off

}

delay_ms(POLL_DELAY_MS);

}

}

## Lab Task 3

#include <stdio.h>

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#include "driver/ledc.h"

#include "esp_err.h"

#define LEDC_TIMER              LEDC_TIMER_0

#define LEDC_MODE               LEDC_LOW_SPEED_MODE

#define LEDC_OUTPUT_IO          (2)                // Standard on-board LED GPIO pin

#define LEDC_CHANNEL            LEDC_CHANNEL_0

#define LEDC_DUTY_RES           LEDC_TIMER_13_BIT // 13-bit resolution (0 to 8191)

#define LEDC_FREQUENCY          (5000)            // Frequency in Hz (5 kHz)

#define LEDC_MAX_DUTY           (8191)            // 2^13 - 1 = 100% duty cycle

#define FADE_TIME_MS            (2000)            // 2 seconds fade time

void app_main(void)

{

ledc_timer_config_t ledc_timer = {

.speed_mode       = LEDC_MODE,

.duty_resolution  = LEDC_DUTY_RES,

.timer_num        = LEDC_TIMER,

.freq_hz          = LEDC_FREQUENCY,

.clk_cfg          = LEDC_AUTO_CLK

};

ledc_timer_config(&ledc_timer);

ledc_channel_config_t ledc_channel = {

.speed_mode     = LEDC_MODE,

.channel        = LEDC_CHANNEL,

.timer_sel      = LEDC_TIMER,

.intr_type      = LEDC_INTR_DISABLE,

.gpio_num       = LEDC_OUTPUT_IO,

.duty           = 0,

.hpoint         = 0

};

ledc_channel_config(&ledc_channel);

ledc_fade_func_install(0);

while (1) {

ledc_set_fade_with_time(LEDC_MODE, LEDC_CHANNEL, LEDC_MAX_DUTY, FADE_TIME_MS);

ledc_fade_start(LEDC_MODE, LEDC_CHANNEL, LEDC_FADE_WAIT_DONE);

ESP_ERROR_CHECK(ledc_set_fade_with_time(LEDC_MODE, LEDC_CHANNEL, 0, FADE_TIME_MS));

ESP_ERROR_CHECK(ledc_fade_start(LEDC_MODE, LEDC_CHANNEL, LEDC_FADE_WAIT_DONE));

}

}

# Lab 2

## Program 1

#include "driver/gpio.h"

#include "driver/uart.h"

#define TXD_PIN GPIO_NUM_17

#define RXD_PIN GPIO_NUM_16

#define UART_NUM UART_NUM_2

#define BAUD 9600

void UART_init(void)

{

const uart_config_t uart_config = {

.baud_rate = BAUD,

.data_bits = UART_DATA_8_BITS,

.parity = UART_PARITY_DISABLE,

.stop_bits = UART_STOP_BITS_1,

.flow_ctrl = UART_HW_FLOWCTRL_DISABLE,

.source_clk = UART_SCLK_DEFAULT,

};

//uart_driver_install(UART_NUM, RX_BUF_SIZE * 2, 0, 0, NULL, 0);

uart_param_config(UART_NUM, &uart_config);

uart_set_pin(UART_NUM, TXD_PIN, RXD_PIN, UART_PIN_NO_CHANGE, UART_PIN_NO_CHANGE);

}

void UART_tx(unsigned char byte)

{

//uint8_t* data = (uint8_t*) malloc(4);

//data = &byte;

uart_write_bytes(UART_NUM, &byte, 1);

//free(data);

}

// Delay for milliseconds

void delay_ms(unsigned int i){

vTaskDelay(i / portTICK_PERIOD_MS);

}

// Delay for microseconds

void delay_us(unsigned int i){

esp_rom_delay_us(i);

}

//******************************************************************************

void app_main(){

UART_init();

for(;;){

UART_tx(0x55);

delay_ms(10);

}

}

## Program 2

#include "driver/gpio.h"

#include "driver/uart.h"

#define UART_NUM UART_NUM_2

#define TXD_PIN GPIO_NUM_17

#define RXD_PIN GPIO_NUM_16

#define BAUD 9600

// Increase buffer size if code does not respond correctly

static const int RX_BUF_SIZE = 256;

void UART_init(void){

const uart_config_t uart_config = {

.baud_rate = BAUD,

.data_bits = UART_DATA_8_BITS,

.parity = UART_PARITY_DISABLE,

.stop_bits = UART_STOP_BITS_1,

.flow_ctrl = UART_HW_FLOWCTRL_DISABLE,

.source_clk = UART_SCLK_DEFAULT,

};

uart_driver_install(UART_NUM, RX_BUF_SIZE * 2, 0, 0, NULL, 0);

uart_param_config(UART_NUM, &uart_config);

uart_set_pin(UART_NUM,TXD_PIN,RXD_PIN,UART_PIN_NO_CHANGE, UART_PIN_NO_CHANGE);

}

unsigned char UART_rx(){

uint8_t data;

//the portMAX_DELAY waits infinitely for reception

uart_read_bytes(UART_NUM, &data, 1, portMAX_DELAY);

return (unsigned char)data;

}

//******************************************************************************

void app_main(){

char in;

UART_init();

for(;;)

in = UART_rx();

}

## In Lab Exercise 1

#include "driver/gpio.h"

#include "driver/uart.h"

#define TXD_PIN GPIO_NUM_17

#define RXD_PIN GPIO_NUM_16

#define UART_NUM UART_NUM_2

#define BAUD 300

#define RX_BUF_SIZE 1024

void UART_init(void)

{

const uart_config_t uart_config = {

.baud_rate = BAUD,

.data_bits = UART_DATA_8_BITS,

.parity = UART_PARITY_DISABLE,

.stop_bits = UART_STOP_BITS_1,

.flow_ctrl = UART_HW_FLOWCTRL_DISABLE,

.source_clk = UART_SCLK_DEFAULT,

};

uart_driver_install(UART_NUM, RX_BUF_SIZE * 2, 0, 0, NULL, 0);

uart_param_config(UART_NUM, &uart_config);

uart_set_pin(UART_NUM, TXD_PIN, RXD_PIN, UART_PIN_NO_CHANGE, UART_PIN_NO_CHANGE);

}

void UART_tx(unsigned char byte)

{

uart_write_bytes(UART_NUM, &byte, 1);

}

// Delay for milliseconds

void delay_ms(unsigned int i){

vTaskDelay(i / portTICK_PERIOD_MS);

}

// Delay for microseconds

void delay_us(unsigned int i){

esp_rom_delay_us(i);

}

//******************************************************************************

void app_main(){

UART_init();

for(;;){

// send a burst of 30 bytes (about 1 second at 300 baud)

for(int n = 0; n < 30; n++){

UART_tx(0x55);

}

delay_ms(1000);   // idle pause, no data

}

}

## In Lab Exercise 2

#include "driver/gpio.h"

#include "driver/uart.h"

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#define UART_NUM   UART_NUM_2

#define TXD_PIN    GPIO_NUM_17

#define RXD_PIN    GPIO_NUM_16

#define BAUD       9600

#define LED_PIN    GPIO_NUM_2      // On-board LED on most ESP32 DevKit boards

static const int RX_BUF_SIZE = 256;

void UART_init(void) {

const uart_config_t uart_config = {

.baud_rate = BAUD,

.data_bits = UART_DATA_8_BITS,

.parity    = UART_PARITY_DISABLE,

.stop_bits = UART_STOP_BITS_1,

.flow_ctrl = UART_HW_FLOWCTRL_DISABLE,

.source_clk = UART_SCLK_DEFAULT,

};

uart_driver_install(UART_NUM, RX_BUF_SIZE * 2, 0, 0, NULL, 0);

uart_param_config(UART_NUM, &uart_config);

uart_set_pin(UART_NUM, TXD_PIN, RXD_PIN, UART_PIN_NO_CHANGE, UART_PIN_NO_CHANGE);

}

unsigned char UART_rx(void) {

uint8_t data;

uart_read_bytes(UART_NUM, &data, 1, portMAX_DELAY);

return (unsigned char)data;

}

void LED_init(void) {

gpio_reset_pin(LED_PIN);

gpio_set_direction(LED_PIN, GPIO_MODE_OUTPUT);

gpio_set_level(LED_PIN, 0);    // LED off initially

}

void app_main(void) {

char in;

UART_init();

LED_init();

for (;;) {

in = UART_rx();

if (in == 'A') {

gpio_set_level(LED_PIN, 1);              // LED ON

vTaskDelay(1000 / portTICK_PERIOD_MS);   // 1 second

gpio_set_level(LED_PIN, 0);              // LED OFF

} else {

gpio_set_level(LED_PIN, 0);              // Any other character: stay OFF

}

}

}

## Lab Task 1

#include <ctype.h>

#include "driver/gpio.h"

#include "driver/uart.h"

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#define UART_NUM      UART_NUM_0

#define TXD_PIN       GPIO_NUM_1      // UART0 TX

#define RXD_PIN       GPIO_NUM_3      // UART0 RX (connect HC-05/06 TX here)

#define BAUD          9600

#define LED_PIN       GPIO_NUM_18

#define FLASH_COUNT   10

#define FLASH_MS      250             // ON time and OFF time per flash

static const int RX_BUF_SIZE = 256;

void UART_init(void) {

const uart_config_t uart_config = {

.baud_rate  = BAUD,

.data_bits  = UART_DATA_8_BITS,

.parity     = UART_PARITY_DISABLE,

.stop_bits  = UART_STOP_BITS_1,

.flow_ctrl  = UART_HW_FLOWCTRL_DISABLE,

.source_clk = UART_SCLK_DEFAULT,

};

uart_driver_install(UART_NUM, RX_BUF_SIZE * 2, 0, 0, NULL, 0);

uart_param_config(UART_NUM, &uart_config);

uart_set_pin(UART_NUM, TXD_PIN, RXD_PIN, UART_PIN_NO_CHANGE, UART_PIN_NO_CHANGE);

}

unsigned char UART_rx(void) {

uint8_t data;

// portMAX_DELAY waits indefinitely for a byte

uart_read_bytes(UART_NUM, &data, 1, portMAX_DELAY);

return (unsigned char)data;

}

void LED_init(void) {

gpio_reset_pin(LED_PIN);

gpio_set_direction(LED_PIN, GPIO_MODE_OUTPUT);

gpio_set_level(LED_PIN, 0);

}

void LED_flash(int times) {

for (int i = 0; i < times; i++) {

gpio_set_level(LED_PIN, 1);

vTaskDelay(FLASH_MS / portTICK_PERIOD_MS);

gpio_set_level(LED_PIN, 0);

vTaskDelay(FLASH_MS / portTICK_PERIOD_MS);

}

}

void app_main(void) {

unsigned char in;

UART_init();

LED_init();

for (;;) {

in = UART_rx();

if (isdigit(in)) {          // '0' to '9' (ASCII 0x30 to 0x39)

LED_flash(FLASH_COUNT);

}

// Any non-digit character is ignored, so the LED stays off

}

}

## Lab Task 2

### TX Board Code

#include "driver/gpio.h"

#include "driver/uart.h"

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#define UART_NUM      UART_NUM_2

#define TXD_PIN       GPIO_NUM_17

#define RXD_PIN       GPIO_NUM_16

#define BAUD          9600

#define LED_PIN       GPIO_NUM_2       // On-board LED

#define STREAM_LEN    64

#define THRESHOLD     32

// Stream 1: 35 lower-case letters (count > 32, LED should turn on)

static const char stream[] =

"the quick brown fox jumps over the lazy dog 0123456789 ESP32 RX!";

// Stream 2: only 3 lower-case letters (count <= 32, LED should stay off)

// To test it, comment out Stream 1 above and use this one instead.

// static const char stream[] =

//     "ESP32 LAB TASK 2: UART LINK BETWEEN TX AND RX BOARDS 1234567 abc";

_Static_assert(sizeof(stream) - 1 == STREAM_LEN, "Stream must be exactly 64 characters");

static const int RX_BUF_SIZE = 256;

void UART_init(void) {

const uart_config_t uart_config = {

.baud_rate  = BAUD,

.data_bits  = UART_DATA_8_BITS,

.parity     = UART_PARITY_DISABLE,

.stop_bits  = UART_STOP_BITS_1,

.flow_ctrl  = UART_HW_FLOWCTRL_DISABLE,

.source_clk = UART_SCLK_DEFAULT,

};

uart_driver_install(UART_NUM, RX_BUF_SIZE * 2, 0, 0, NULL, 0);

uart_param_config(UART_NUM, &uart_config);

uart_set_pin(UART_NUM, TXD_PIN, RXD_PIN, UART_PIN_NO_CHANGE, UART_PIN_NO_CHANGE);

}

void LED_init(void) {

gpio_reset_pin(LED_PIN);

gpio_set_direction(LED_PIN, GPIO_MODE_OUTPUT);

gpio_set_level(LED_PIN, 0);

}

void app_main(void) {

uint8_t count = 0;

UART_init();

LED_init();

for (;;) {

// Clear any stale bytes, then send the 64-character stream

uart_flush_input(UART_NUM);

uart_write_bytes(UART_NUM, stream, STREAM_LEN);

uart_wait_tx_done(UART_NUM, pdMS_TO_TICKS(500));

// Wait up to 2 seconds for the RX board to reply with the count

int len = uart_read_bytes(UART_NUM, &count, 1, pdMS_TO_TICKS(2000));

if (len == 1 && count > THRESHOLD) {

gpio_set_level(LED_PIN, 1);               // LED ON for 1 second

vTaskDelay(pdMS_TO_TICKS(1000));

gpio_set_level(LED_PIN, 0);

} else {

gpio_set_level(LED_PIN, 0);               // LED OFF

}

vTaskDelay(pdMS_TO_TICKS(2000));              // Pause before the next round

}

}

### RX Board Code

#include <ctype.h>

#include "driver/gpio.h"

#include "driver/uart.h"

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#define UART_NUM      UART_NUM_2

#define TXD_PIN       GPIO_NUM_17

#define RXD_PIN       GPIO_NUM_16

#define BAUD          9600

#define STREAM_LEN    64

static const int RX_BUF_SIZE = 256;

void UART_init(void) {

const uart_config_t uart_config = {

.baud_rate  = BAUD,

.data_bits  = UART_DATA_8_BITS,

.parity     = UART_PARITY_DISABLE,

.stop_bits  = UART_STOP_BITS_1,

.flow_ctrl  = UART_HW_FLOWCTRL_DISABLE,

.source_clk = UART_SCLK_DEFAULT,

};

uart_driver_install(UART_NUM, RX_BUF_SIZE * 2, 0, 0, NULL, 0);

uart_param_config(UART_NUM, &uart_config);

uart_set_pin(UART_NUM, TXD_PIN, RXD_PIN, UART_PIN_NO_CHANGE, UART_PIN_NO_CHANGE);

}

void app_main(void) {

uint8_t buffer[STREAM_LEN];

uint8_t count;

UART_init();

for (;;) {

// Wait up to 1 second for a complete 64-byte stream

int len = uart_read_bytes(UART_NUM, buffer, STREAM_LEN, pdMS_TO_TICKS(1000));

if (len == STREAM_LEN) {

count = 0;

for (int i = 0; i < STREAM_LEN; i++) {

if (islower(buffer[i])) {     // 'a' to 'z'

count++;

}

}

// Send the count back to the TX board as a single byte

uart_write_bytes(UART_NUM, (const char *)&count, 1);

uart_wait_tx_done(UART_NUM, pdMS_TO_TICKS(100));

} else if (len > 0) {

// Incomplete stream: discard it and resynchronize

uart_flush_input(UART_NUM);

}

}

}

# Lab 3

## Program 1

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#include "driver/adc.h"

#include "esp_adc_cal.h"

//ADC settings

#define DEFAULT_VREF 1100 // Default reference voltage (in mV) for ESP32

#define NO_OF_SAMPLES 8 // Multisampling

#define ADC_CHANNEL ADC1_CHANNEL_6 // ADC1 channel connected to GPIO34

// Delay for milliseconds

void delay_ms(unsigned int ms){

vTaskDelay(pdMS_TO_TICKS(ms));

}

//function to read average value of multi-conversion result

uint16_t ADC_result(uint8_t samples){

uint16_t adc = 0;

for (uint8_t i = 0; i < samples; i++) {

adc += adc1_get_raw(ADC_CHANNEL);

}

adc /= samples; // Average the samples

return(adc);

}

/********************************************************************************/

void app_main() {

uint16_t ADC = 0;

// Initialize the ADC

adc1_config_width(ADC_WIDTH_BIT_12); // 12-bit ADC width

adc1_config_channel_atten(ADC_CHANNEL, ADC_ATTEN_DB_12);

for(;;){

ADC = ADC_result(8);

delay_ms(200);

}

}

## In Lab Exercise 1

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#include "driver/adc.h"

#include "driver/gpio.h"

#include "esp_adc_cal.h"

//ADC settings

#define DEFAULT_VREF 1100   // Default reference voltage (in mV) for ESP32

#define NO_OF_SAMPLES 8     // Multisampling

#define ADC_CHANNEL ADC1_CHANNEL_6

//LED settings

#define LED_PIN GPIO_NUM_2  // Onboard LED

#define ADC_LOW_THRESHOLD  1000

#define ADC_HIGH_THRESHOLD 2500

// Delay for milliseconds

void delay_ms(unsigned int ms){

vTaskDelay(pdMS_TO_TICKS(ms));

}

//function to read average value of multi-conversion result

uint16_t ADC_result(uint8_t samples){

uint16_t adc = 0;

for (uint8_t i = 0; i < samples; i++) {

adc += adc1_get_raw(ADC_CHANNEL);

}

adc /= samples;  // Average the samples

return(adc);

}

void app_main() {

uint16_t ADC = 0;

// Initialize the ADC

adc1_config_width(ADC_WIDTH_BIT_12);  // 12-bit ADC width

adc1_config_channel_atten(ADC_CHANNEL, ADC_ATTEN_DB_12);

// Initialize the LED pin as output

gpio_reset_pin(LED_PIN);

gpio_set_direction(LED_PIN, GPIO_MODE_OUTPUT);

for(;;){

ADC = ADC_result(8);

if (ADC >= ADC_LOW_THRESHOLD && ADC <= ADC_HIGH_THRESHOLD) {

gpio_set_level(LED_PIN, 1);  // LED ON

} else {

gpio_set_level(LED_PIN, 0);  // LED OFF

}

delay_ms(200);

}

}

## Lab Task 1

#include <stdio.h>

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#include "driver/adc.h"

#include "driver/gpio.h"

#include "esp_adc_cal.h"

#define PRESET_TEMP_C      30.0f            // software pre-set temperature (°C)

#define DEFAULT_VREF       1100             // mV

#define NO_OF_SAMPLES      16               // multisampling for a stable reading

#define ADC_CHANNEL        ADC2_CHANNEL_0   // GPIO4 (ADC2_CH0)

#define ADC_ATTEN          ADC_ATTEN_DB_0   // ~0 to 1.1 V range

#define ADC_WIDTH          ADC_WIDTH_BIT_12

#define LED_PIN            GPIO_NUM_2       // on-board LED

static esp_adc_cal_characteristics_t adc_chars;

void delay_ms(unsigned int ms)

{

vTaskDelay(pdMS_TO_TICKS(ms));

}

uint32_t ADC_result(uint8_t samples)

{

uint32_t sum = 0;

int raw = 0;

uint8_t valid = 0;

for (uint8_t i = 0; i < samples; i++) {

if (adc2_get_raw(ADC_CHANNEL, ADC_WIDTH, &raw) == ESP_OK) {

sum += raw;

valid++;

}

}

return (valid > 0) ? (sum / valid) : 0;

}

void app_main(void)

{

// ADC2 initialisation

adc2_config_channel_atten(ADC_CHANNEL, ADC_ATTEN);

esp_adc_cal_characterize(ADC_UNIT_2, ADC_ATTEN, ADC_WIDTH, DEFAULT_VREF, &adc_chars);

// LED initialisation

gpio_reset_pin(LED_PIN);

gpio_set_direction(LED_PIN, GPIO_MODE_OUTPUT);

gpio_set_level(LED_PIN, 0);

for (;;) {

uint32_t raw = ADC_result(NO_OF_SAMPLES);

uint32_t mv  = esp_adc_cal_raw_to_voltage(raw, &adc_chars);  // calibrated mV

float temp_c = mv / 10.0f;                                   // LM35: 10 mV per °C

if (temp_c > PRESET_TEMP_C) {

gpio_set_level(LED_PIN, 1);   // above pre-set -> LED ON

} else {

gpio_set_level(LED_PIN, 0);   // otherwise -> LED OFF

}

printf("Raw: %lu | Voltage: %lu mV | Temp: %.1f C | Preset: %.1f C | LED: %s\n",

(unsigned long)raw, (unsigned long)mv, temp_c, PRESET_TEMP_C,

(temp_c > PRESET_TEMP_C) ? "ON" : "OFF");

delay_ms(500);

}

}

## Lab Task 2

#include <stdio.h>

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#include "driver/adc.h"

#include "driver/gpio.h"

#include "esp_adc_cal.h"

#define DEFAULT_VREF     1100                 // mV

#define NO_OF_SAMPLES    32                   // multiple conversions for averaging

#define ADC_WIDTH        ADC_WIDTH_BIT_12

#define SET_CHANNEL      ADC1_CHANNEL_1       // preset (potentiometer) -> GPIO37

#define LM35_CHANNEL     ADC1_CHANNEL_2       // LM35 sensor            -> GPIO38

#define SET_ATTEN        ADC_ATTEN_DB_12      // pot swings 0 to 3.3 V -> full range

#define LM35_ATTEN       ADC_ATTEN_DB_0       // LM35 is < 1.1 V -> best resolution

#define PRESET_MAX_C     100.0f

#define LED_PIN          GPIO_NUM_2           // on-board LED

static esp_adc_cal_characteristics_t lm35_chars;

void delay_ms(unsigned int ms)

{

vTaskDelay(pdMS_TO_TICKS(ms));

}

uint32_t ADC_result(adc1_channel_t channel, uint8_t samples)

{

uint32_t sum = 0;

for (uint8_t i = 0; i < samples; i++) {

sum += adc1_get_raw(channel);

}

return sum / samples;

}

void app_main(void)

{

// ADC1 initialization

adc1_config_width(ADC_WIDTH);

adc1_config_channel_atten(SET_CHANNEL,  SET_ATTEN);

adc1_config_channel_atten(LM35_CHANNEL, LM35_ATTEN);

// Calibration profile for the LM35 channel

esp_adc_cal_characterize(ADC_UNIT_1, LM35_ATTEN, ADC_WIDTH, DEFAULT_VREF, &lm35_chars);

// LED initialization

gpio_reset_pin(LED_PIN);

gpio_set_direction(LED_PIN, GPIO_MODE_OUTPUT);

gpio_set_level(LED_PIN, 0);

for (;;) {

// 1) Pre-set temperature from potentiometer (channel 1)

uint32_t set_raw = ADC_result(SET_CHANNEL, NO_OF_SAMPLES);

float preset_c   = (set_raw * PRESET_MAX_C) / 4095.0f;

// 2) Measured temperature from LM35 (channel 2), averaged

uint32_t lm_raw  = ADC_result(LM35_CHANNEL, NO_OF_SAMPLES);

uint32_t lm_mv   = esp_adc_cal_raw_to_voltage(lm_raw, &lm35_chars);

float temp_c     = lm_mv / 10.0f;         // LM35: 10 mV per C

// 3) Compare and drive LED

if (temp_c > preset_c) {

gpio_set_level(LED_PIN, 1);           // above preset -> ON

} else {

gpio_set_level(LED_PIN, 0);           // otherwise    -> OFF

}

printf("Preset: %.1f C | Temp: %.1f C (%lu mV) | LED: %s\n",

preset_c, temp_c, (unsigned long)lm_mv,

(temp_c > preset_c) ? "ON" : "OFF");

delay_ms(300);

}

}